International Conference on Applied Energy 2021

Nov. 29 - Dec. 2, 2021, in Bangkok, Thailand

# PyPSA-Brazil: Designing a Brazilian energy system model for studying energy planning at high and temporal resolution

Ying Deng1, Karl-Kiên Cao 1*, Bruna Wanke12*

1 Institute of Networked Energy Systems, Energy Systems Analysis, German Aerospace Center, Germany

2 Institute for Building Energetics, Thermotechnology and Energy Storage, Universität Stuttgart

# SUPPLEMENTARY

Table A- 1: Technology cost and lifetime assumptions based on 2050 (variable operation and maintenance – VOM, Fixed Operation and Maintenance -FOM)

| technology | parameter | value | unit | source |
| --- | --- | --- | --- | --- |
| AC linea | investment | 6286,544 | R$/MW/km | (ANEEL 2021) |
|  | FOM | 2 | %/year | (Hagspiel et al. 2014) |
|  | lifetime | 40 | years | (Hagspiel et al. 2014) |
| HVDC line | investment | 436,6 | R$/MW/km | (ANEEL 2021) |
|  | FOM j | 2 | %/year | (Hagspiel et al. 2014) |
|  | lifetime | 40 | years | (Hagspiel et al. 2014) |
| Lithium Ion Battery | charge efficiency b | 0,985 | per unit | (DEA 2020b) |
|  | discharge efficiency b | 0,975 | per unit | (DEA 2020b) |
|  | max hour c | 0,33 | h | (DEA 2020b) |
|  | loss d | 0,004 | per unit | (DEA 2020b) |
|  | lifetime e | 30 | years | (DEA 2020b) |
|  | FOM f | 0,9 | %/year | (DEA 2020b) |
|  | VOM | 1,6 | €/MWh | (DEA 2020b) |
|  | investment g | 60000 | €/MW | (DEA 2020b) |
| Onshore Wind | FOM h | 1,2 | %/year | (DEA 2020a) |
|  | VOM | 1,22 | €/MWh | (DEA 2020a) |
|  | investment i | 960000 | €/MW | (DEA 2020a) |
|  | lifetime e | 30 | years | (DEA 2020a) |
| Solar PV | FOM j | 1,6 | %/year | (DEA 2020a) |
|  | VOM k | 0,01 | €/MWh | (Brown et al. 2018) |
|  | investment l | 490000 | €/MW | (DEA 2020a) |
|  | lifetime e | 40 | years | (DEA 2020a) |
| Hydro Reservoir | FOM | 2 | %/year | (Schröder et al. 2013) |
|  | efficiency | 0,9 | per unit | (Schröder et al. 2013) |
|  | investment m | 1565000 | €/MW | (Schröder et al. 2013) |
|  | lifetime | 80 | years | (Brown et al. 2018) |
|  | max hour | 18 | h | (ONS 2021). |
| Offshore Wind | FOM n | 1,8 | %/year | (DEA 2020a) |
|  | VOM n | 2,4 | €/MWh | (DEA 2020a) |
|  | investment o | 1780000 | €/MW | (DEA 2020a) |
|  | lifetime e | 30 | years | (DEA 2020a) |
| - this is the average value of all projects listed in the source. The transmission capacity is denoted by the capacity of the converter. If more than one capacity exists, the smallest is selected. The voltage difference is ignored. - In sheet - 180 lithium ion battery, the value = round trip efficiency DC/ (discharge) charging efficiency (%). - the value = Energy storage capacity for one unit (MWh)/Output capacity for one unit (MW) - the value = Energy losses during storage(%/day)/24 - refers to “Technical lifetime” - the value = Fixed O&M / Output capacity expansion cost investment * 100 - refers to “Output capacity expansion cost investment” - in sheet - 20 Large wind turbines on land, the value = Fixed O&M/Nominal investment - refers to “Nominal investment” - In sheet - 22 Photovoltaics Medium, the value = Fixed O&M/Specific investment, total system - 0.01 is to fix curtailment order of renewable generation technology - In sheet - 22 Photovoltaics Medium, refers to “Specific investment, total” - refers to “reservoir repowered (2.5 Hydro/ page 27)” - in sheet - 21 Large wind turbines off-shore, refers to Fixed O&M / Variable O&M - the value = investment costs - Nominal investment grid connection costs |  |  |  |  |

- Reference

- ANEEL (2021): Transmission Auction Results. raw data. Available online at https://www.aneel.gov.br/documents/654791/20589082/Resultado+do+Leil%C3%A3o+de+Transmiss%C3%A3o/0ac53811-f890-9274-1ba9-4bee7cab9723, checked on 5/20/2021.

- Brown, T.; Schlachtberger, D.; Kies, A.; Schramm, S.; Greiner, M. (2018): Synergies of sector coupling and transmission reinforcement in a cost-optimised, highly renewable European energy system. In Energy 160, pp. 720–739. DOI: 10.1016/j.energy.2018.06.222.

- DEA (2020a): Technology Data for Energy Plants for Electricity and District Heating Generation. raw data. Available online at https://ens.dk/en/our-services/projections-and-models/technology-data/technology-data-generation-electricity-and, checked on 1/5/2021.

- DEA (2020b): Technology Data for Energy Storage. Danish Energy Agency and Energinet. Available online at https://ens.dk/en/our-services/projections-and-models/technology-data/technology-data-energy-storage, checked on 1/5/2021.

- Hagspiel, S.; Jägemann, C.; Lindenberger, D.; Brown, T.; Cherevatskiy, S.; Tröster, E. (2014): Cost-optimal power system extension under flow-based market coupling. In Energy 66, pp. 654–666. DOI: 10.1016/j.energy.2014.01.025.

- ONS (2021): Reserva Girante na Demanda Máxima do SIN. Brasil. Available online at http://sdro.ons.org.br/SDRO/DIARIO/2021_06_28/HTML/13_ReservaGiranteDemandaMaxima.html, checked on 6/28/2021.

- Schröder, Andreas; Kunz, Friedrich; Meiss, Jan; Mendelevitch, Roman; Hirschhausen, Christian von (2013): Current and prospective costs of electricity generation until 2050. DIW data documentation.